FTDX-10 Rig Control Version 1 — Setup & User Guide

Version 1 Notice: This is Version 1 build shared for testing purposes. Please report any bugs, unexpected behavior, or suggestions. Your feedback directly shapes the next release.

# 1.  What’s in the Folder

| File | Purpose |
| --- | --- |
| FTDX10RigControlVersion1.exe | The program. Double-click to run — no installer needed. |
| FTDX10_Setup.docx | This document. |
| rig_Control_Settings.json | Created automatically on first run. Saves your COM port, baud rate, and slider positions between sessions. |
| rig_memories.json | Created automatically on first run. Stores your 30 memory / favorite slots. |
| rig_log.csv | Created automatically on first run. Rolling 30-day log of all events, errors, and CAT traffic (when enabled). |
| FTDX10_Tips.docx | Optional tips and tricks document. |

Tip: Keep all files together in the same folder. The program reads and writes rig_Control_Settings.json, rig_memories.json, and rig_log.csv from whichever folder the .exe lives in.

# 2. Hardware & Driver Setup

Connect the radio to your PC with a USB cable before launching the program. Windows should install the Silicon Labs CP210x driver automatically.

## 2.1 Verify the COM Port in Device Manager

Open Windows Device Manager and expand Ports (COM & LPT). Look for:

Look for: Silicon Labs CP210x USB to UART Bridge "Enhanced" COM Port (COM X) — write down the COM number.

The Enhanced port is the CAT control port. There is also a Standard port — do not use it for this program.

## 2.2 Disable Power Management on the COM Port

Windows can turn off USB devices to save power, which drops the CAT connection mid-session. Prevent this:

- Right-click on the Enhanced COM Port entry.

- Select Properties, then click the Power Management tab.

- Uncheck “Allow the computer to turn off this device to save power.”

- Click OK.

## 2.3 Verify COM Port Speed Settings

Right-click the Enhanced COM Port → Properties → Port Settings tab. Confirm these values:

| Setting | Required Value |
| --- | --- |
| Bits per second | 38400 |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |

## 2.4 Verify Audio Device

In Device Manager expand Sound, Video and Game Controllers. Confirm you see:

Look for: USB Audio CODEC — this confirms the radio’s audio interface is active.

## 2.5 Check for Conflicts

Before launching, confirm in Device Manager:

- No duplicate COM port entries

- No disabled devices (shown as greyed-out)

- No yellow warning triangles on any USB or COM entry

# 3.  First Launch

- Double-click FTDX10RigControlVersion1.exe. No installer is required.

- In the Live (Selected / Unselected VFO) panel at the top left, select your COM port from the dropdown (the Enhanced port you noted earlier).

- Set Baud to 38400.

- Make sure the RTS checkbox is checked.

- Click Connect. The status label will change to Connected: COMX when successful.

Note: If the program was run before with a saved COM port, it will attempt to auto-connect on startup. If the radio is on and the cable is connected, it connects automatically with no action needed.

# 4. Program Layout

The window is divided into two columns. Everything on the left is live radio status and VFO/memory control. Everything on the right is for setting targets, levels, and diagnostics.

## Left Column

| Panel | What it does |
| --- | --- |
| Live (Selected / Unselected VFO) | Shows the real-time frequency, mode, and band for both VFOs. Also contains the COM port connection controls, S-meter bar, and RX/TX button. |
| VFO Control | Select VFO A or B as the active receiver. Swap A↔B exchanges both VFOs. Copy A→B copies VFO A’s frequency and mode into VFO B. |
| Memories (Favorites) | 30 user-programmable memory slots. Save a frequency/mode/band/filter combo, recall it to the radio, or delete it. The list shows all named slots. |
| Split | Enable split TX/RX operation, set a TX offset, and monitor RF and TX frequencies. XFC (cross-VFO check) momentarily tunes the receiver to the TX frequency. |

## Right Column

| Panel | What it does |
| --- | --- |
| Frequency / Mode / Band | Type in a band, frequency, and mode then click Set Freq, Set Mode, Set Band, or Set All to send them to the active VFO. The ▲/▼ buttons and step selector let you fine-tune the frequency in small increments. |
| Levels | Real-time sliders for Volume, Squelch, RF Power, Mic Gain, and RF Gain. Changes are sent live as you drag. |
| TX Meters | Power, SWR, and ALC bars update while transmitting. |
| Quick Buttons | One-click DATA, RTTY, SSB, and CW mode buttons. Each click cycles between upper and lower variants (e.g. USB ↔ LSB, CW-U ↔ CW-L). |
| Frequency Scanner | Scans a frequency range on the active VFO, pausing when signal strength meets the Stop-at threshold. Start, Pause, and Stop controls provided. |
| TX Timeout Safety | Automatically drops MOX if the radio transmits longer than the configured timeout. Enable the checkbox and set the seconds. |
| Log | On-screen event log with diagnostic toggles (see Section 6). |

# 5. VFO Controls

The VFO Control panel has four buttons. Each does a full re-apply of frequency, band, and mode to ensure the radio’s receiver chain is fully tuned — not just a logical switch.

| Button | What it does |
| --- | --- |
| Select VFO A | Reads VFO A’s current frequency and mode, switches the active VFO to A, then re-applies that frequency, band, and mode to VFO A so audio follows immediately. |
| Select VFO B | Same as above but for VFO B. |
| Swap A↔B | Exchanges the frequency and mode of both VFOs on the radio, then refreshes the display. |
| Copy A→B | Copies VFO A’s frequency, band, and mode into VFO B, then refreshes the display. |

After Scanning: If you use the Frequency Scanner and stop on a signal, always click Select VFO A or Select VFO B afterward to fully re-engage that VFO’s receiver chain. This ensures audio comes from the correct VFO.

# 6. Log & Diagnostics

The Log panel shows timestamped events in the on-screen text box and writes them to rig_log.csv (kept for 30 days). Four buttons control what gets logged:

| Button | What it logs |
| --- | --- |
| Log TX Meters | Logs raw RM meter readings (power, SWR, ALC) and TX/MOX state every 1.5 seconds while transmitting. Off by default. Turn on when debugging TX issues. |
| Log CAT Traffic | Logs every CAT command sent to the radio and the reply received, e.g. FA; → FA007204000;. Use this when something is behaving unexpectedly to see exactly what is happening on the serial port. Off by default — generates a lot of entries. |
| Log Heartbeat | Logs a ‘Polling heartbeat’ line every 5 seconds to confirm the background poll loop is running. Useful if the display appears frozen. Off by default. |
| Clear Log | Clears the on-screen log text box only. Does not delete the rig_log.csv file. |

When reporting a bug: Turn on Log CAT Traffic, reproduce the problem, then open rig_log.csv and paste the relevant lines into your bug report. This is the fastest way to diagnose what went wrong.

# 7. Troubleshooting

## Won’t Connect / “Disconnected”

- Verify you selected the Enhanced COM port (not Standard).

- Confirm baud is 38400 and RTS is checked.

- Check Device Manager for yellow warning icons on the COM port.

- Disable power management on the COM port (see Section 2.2).

- Try unplugging and re-plugging the USB cable, then refreshing the port list.

- Make sure no other program (HRD, WSJT-X, etc.) has the port open.

## Display Freezes / Stops Updating

- Enable Log Heartbeat. If heartbeat entries stop appearing, the poll loop has crashed — disconnect and reconnect.

- Check the Log panel for a “Polling stopped” error message.

- Ensure no other CAT program is competing for the same COM port.

## Audio Coming from Wrong VFO After Scan

- Click Select VFO A or Select VFO B after stopping a scan. This re-applies the frequency and mode to force the receiver chain to the correct VFO.

## Frequency Not Changing

- Check that the Frequency / Mode (TARGET) section has the correct frequency entered, then click Set All.

- If the radio is in Split mode, the inactive VFO may be what is being written. Disable Split first.

## Settings Not Saving Between Sessions

- Confirm rig_Control_Settings.json is in the same folder as the .exe and is not read-only.

- Click Set Defaults before closing the program to force a save.

## TX Timeout Tripping Unexpectedly

- Check the TX Timeout Safety panel. Increase the timeout seconds or uncheck Enable TX Timeout if you are doing long digital transmissions.

# 8. Version 1  Feedback

Thank you for testing! When you find something to report, the most useful information to include is:

- What you were doing when the problem occurred (e.g. stopped a scan, clicked Select VFO A).

- What you expected to happen.

- What actually happened.

- The relevant lines from rig_log.csv (enable Log CAT Traffic before reproducing the issue for maximum detail).

- Your radio firmware version if known (shown on the FTDX-10 front panel menu).

- If you need help, email me at 5xjranch@gmail.com

Known Limitation: There is a brief ~0.3 second delay when switching VFOs using the Select buttons. This is intentional — the program reads the VFO, switches, and re-applies the frequency and mode to guarantee audio follows the selection. It will be optimized in a future release.