# IC‑7300 Rig Control – Setup Guide

## IC‑7300 Connection Checklist

Before running the program, verify the following items:

- Install the USB drivers from the ICOM Japan website (recommended).
2. Connect the radio to the computer using the USB cable.
3. Open Windows Device Manager.

## Verify the COM Port

In Device Manager expand: Ports (COM & LPT)

Look for:
Silicon Labs CP210x USB to UART Bridge (COMx)

The COM number may be different on your computer. Write this number down because the program must be configured to use the same COM port.

## Verify the Audio Device

In Device Manager expand: Sound, Video and Game Controllers

Look for:
USB Audio CODEC

This confirms the radio audio interface is working.

## Check Power Management (Important)

Windows sometimes turns off USB devices to save power.

Go to:
Device Manager → Ports → Properties → Power Management

Uncheck:
'Allow the computer to turn off this device to save power'

## Verify COM Port Settings

Device Manager → Ports → Properties → Port Settings

Recommended settings:
Bits per second: 115200
Data bits: 8
Parity: None
Stop bits: 1
Flow control: None

## Check for Conflicts

In Device Manager confirm:
• No duplicate COM ports
• No disabled devices
• No yellow warning icons